Software Test Document (STD)

CircuitSense

Group Name: Queue5

Team Members:

- Jaden Cirius

- Brooke Zally

- Angel Martinez

- Charles-Olivier Dorcent

- Eric Pinheiro

Course:
CEN 5035-001, CEN 4010-002: Software Engineering

Instructor:
Safak Kayikci

Date:
July 26, 2026

Table of Contents

1. Test Plan
    1.1 Introduction
    1.2 Test Items
    1.3 Features to Be Tested
    1.4 Features Not to Be Tested
    1.5 Test Approach
    1.6 Pass/Fail Criteria
    1.7 Suspension and Resumption Criteria

2. Test Design Specification
    2.1 Test Case Structure and Conventions
    2.2 Test Case Index
    2.3 Host Companion Application Test Cases
    2.4 Communication Layer Test Cases
    2.5 Device Display Module Test Cases
    2.6 Non-Functional Test Cases
    2.7 Test Procedures

3. Test Execution Report
    3.1 Test Results
    3.2 Defect Report
    3.3 Requirements Traceability Matrix

4. Test Summary
    4.1 Test Environment
    4.2 Overall Test Results
    4.3 Conclusion

- Test Plan

This section defines the test plan for the CIrcuitSense system in accordance with IEEE STF 829-1998, the IEEE standard for Software Test Documentation. It establishes the purpose, scope, and objectives of the testing effort. It also identifies the items and features under test, describes the overall test approach and methodology, and defines the criteria that determines whether testing is considered successful, suspended , or complete.

  - Introduction

Purpose

The purpose of this Software Test Document is to define and guide the testing activities for the CircuitSense hardware monitoring system. CircuitSense is a standalone PC hardware monitoring platform consisting of a host companion application (Circuit Sense.py) running on a Windows or Linux PC, and a Raspberry Pi Zero 2 W device with an attached 3.5-inch SPI TFT display that presents real-time system metrics to the user.

This STD serves as the primary reference for verifying that the CircuitSense system meets the functional and non-functional requirements defined in the CircuitSense Software Requirements Specification (SRS) and behave as described in the Software Design Document (SDD). It is intended for use by the development team during implementation, by testers executing the test cases, and by the course instructor and graders evaluating the project.

Scope

This test plan covers the complete software testing effort for CircuitSense Version 1.0, encompassing all three primary software components as defined in the SDD:

- The Host Companion Application (Circuit Sense.py) - including OS detection, metric collection (CPU, GPU, RAM, disk, network, uptime), JSON serialization, configuration management, and connection handling.

- The Communication Layer - including both USB serial (CDC-ACM) and Wi-Fi socket transport modes, message framing, and connection-loss behavior.

- The Device Display Module (Pi-side script) - including JSON parsing, threshold evaluation, display rendering, state management, and disconnection handling.

Testing is scoped to the two supported host operating systems: Windows 10/11 and Linux (Ubuntu/Debian-based distributions). macOS is explicitly out of scope for this version.

Objectives

The primary objectives of this testing effort are:

- Verify that all six hardware metrics (CPU usage/temperature, GPU usage/temperature, RAM, disk, network, and uptime) are collected accurately and within acceptable tolerance of reference tools on both Windows and Linux.

- Confirm that all metric data is correctly packaged into the JSON data contract defined in SDD Section 3.4 and transmitted to the CircuitSense device without loss or corruption.

- Validate that the device display renders all metrics correctly, applies color-coded thresholds accurately, and transitions between display states (CONNECTED, DISCONNECTED, STARTING) as specified.

- Verify that both USB serial and Wi-Fi communication modes establish connection, transmit data reliably, and recover gracefully from connection loss.

- Confirm that the first-run setup flow correctly creates config.json and that subsequent launches reconnect automatically without user input.

- Identify and document any defects, deviations from specification, or behaviors that fall outside defined pass/fail criteria before the final project submission.

  - Test Items

The following software items are subject to testing under this plan. Each item corresponds to a component or module described in the CircuitSense SDD.

| Item ID | Item Name | Component | Description |
| --- | --- | --- | --- |
| TI-01 | OS Detection Module | Host Application | Detects host OS at startup and routes to correct library set |
| TI-02 | CPU Metric Collector | Host Application | Reads CPU usage % and temperature via psutil and OS-specific libraries |
| TI-03 | GPU Metric Collector | Host Application | Reads GPU usage % and temperature via LibreHardwareMonitor, pynvml, or sysfs |
| TI-04 | RAM Metric Collector | Host Application | Reads RAM used, total, and usage % via psutil |
| TI-05 | Disk Metric Collector | Host Application | Reads primary disk usage % via psutil |
| TI-06 | Network Metric Collector | Host Application | Reads upload and download throughput in Mbps via psutil |
| TI-07 | Uptime Collector | Host Application | Reads system uptime in seconds and formats as human-readable string |
| TI-08 | JSON Serializer | Host Application | Packages all metrics into the JSON data contract defined in SDD Section 3.4 |
| TI-09 | Configuration Manager | Host Application | Handles first-run setup prompt and reads/writes config.json |
| TI-10 | USB Serial Transport | Communication Layer | Transmits JSON messages over CDC-ACM USB serial connection |
| TI-11 | Wi-Fi Socket Transport | Communication Layer | Transmits JSON messages over local network TCP socket |
| TI-12 | JSON Parser | Device Display Module | Receives and parses incoming JSON messages on the Pi |
| TI-13 | Threshold Evaluator | Device Display Module | Applies green/yellow/red color thresholds to each metric value |
| TI-14 | Display Renderer | Device Display Module | Renders metric layout to the 3.5-inch TFT display via framebuffer |
| TI-15 | Display State Manager | Device Display Module | Manages transitions between STARTING, CONNECTED, and DISCONNECTED states |

  - Features to Be Tested

The following features and functional behaviors will be tested as part of this plan. Each feature maps to one or more requirements in the CircuitSense SRS

Host Companion Application Features

  - CPU metric collection: accurate reading of CPU usage percentage and CPU temperature on both Windows and Linux, within the tolerance defined in Section 1.6.

  - GPU metric collection: accurate reading of GPU usage percentage and GPU temperature via the correct OS and vendor-specific library, graceful reporting of unavailability when no supported GPU is detected.

  - RAM metric collection: accurate reading of RAM used (GB), total RAM (GB), and RAM usage percentage.

  - Disk metric collection: accurate reading of primary disk usage percentage.

  - Network metric collection: accurate reading of current upload and download throughput in Mbps.

  - Uptime collection: accurate reading of system uptime in seconds; correct formatting as a human-readable string (e.g., '2d 4h 31m').

  - OS detection: correct identification of host OS at startup and routing to the appropriate library set without manual configuration.

  - JSON serialization: all metric values correctly packaged into the schema defined in SDD Section 3.4, with correct field names, data types, and units.

  - First-run configuration: correct presentation of the USB/Wi-Fi mode selection prompt; correct creation and persistence of config.json.

  - Automatic reconnection: on subsequent launches, the application reads config.json and connects automatically without prompting the user.

Communication Layer Features

  - USB serial connection: successful establishment of a CDC-ACM serial connection between the host and the Pi over USB; correct transmission of JSON messages at the specified rate.

  - Wi-Fi socket connection: successful establishment of a TCP socket connection between host and Pi on the same local network, correct transmission of JSON messages.

  - Message framing: correct application and parsing of newline delimiters on both sides of the connection.

  - Connection loss detection: correct detection of timeout or disconnection on both host and device sides within the defined timeout window.

  - Reconnection behavior: correct recovery after a connection drop, including the device re-entering its waiting state and the host attempting to reconnect.

Device Display Module Features

  - JSON parsing: correct parsing of all incoming JSON messages; graceful handling of malformed or incomplete messages without crashing.

  - Metric display accuracy: values shown on the TFT display match the values transmitted in the JSON payload.

  - Color threshold display: correct application of green, yellow, and red indicators based on the threshold constants defined in SDD Section 3.3.

  - GPU unavailability display: correct rendering of 'N/A' in grey when gpu.available is false.

  - Display refresh rate: display updates within the specified 1-2 second interval.

  - STARTING state: correct display of a waiting/startup screen before the first message is received.

  - DISCONNECTED state: correct transition to and display of the disconnected screen when no data is received within the timeout window.

  - CONNECTED state: correct transition back to normal metric display when data resumes after a disconnection.

  - Features Not to be Tested

The following items are explicitly excluded from under this plan, along with the rationale for each exclusion.

  - macOS host compatibility - macOS is out of scope for this version of CircuitSense as documented in the SRS. No macOS-specific sensor libraries are included in the design.

  - Multi-host monitoring - CircuitSense is designed to monitor a single host at a time. Connecting multiple hosts simultaneously is not a supported or tested configuration.

  - Physical hardware durability - stress testing of the 3D-printed enclosure, display glass, or USB connector under physical force or environmental conditions is outside the software testing scope.

  - Network security and encryption - the Wi-Fi socket transport operates without TLS or authentication. Security testing of the communication channel is out of scope for this academic project.

  - LibreHardwareMonitor internal behavior — the internal correctness of LibreHardwareMonitor as a third-party library is assumed; testing is limited to verifying that CircuitSense correctly invokes and handles its output.

  - Raspberry Pi OS and kernel behavior — the correctness of the Pi OS, USB gadget kernel modules, and framebuffer driver are assumed; testing is limited to verifying CircuitSense's interaction with these components.

  - Long-duration (24+ hour) stability testing — extended reliability testing is outside the project timeline. A sustained 30-minute run test is included as the longest duration test under this plan.

  - Test Approach

Testing Levels

Testing will be conducted across three levels, and applies progressively during the development:

  - Unit Testing - individual functions and modules are tested in isolation on the developer's laptop before integration. Each metric collector (CPU, GPU, RAM, disk, network, uptime) is tested independently by running the relevant function and comparing output to a known reference tool. Unit tests do not require the Raspberry Pi to be connected.

  - Integration Testing - the host application and communication layer are tested together, verifying that metric data flows correctly from collection through JSON serialization to transmission. Integration tests verify that the JSON payload received by a test listener matches the values collected by the host script. The Pi display module is also integrated and tested as a receiver at this stage.

  - System Testing - the complete CircuitSense system (host script + communication layer + Pi display) is tested end-to-end. System tests verify that values appearing on the TFT display match the actual host system state, that both USB and Wi-Fi modes function correctly, and that the system behaves correctly under simulated real-world conditions including connection loss and metric boundary values.

Testing Methodologies

  - Black-box testing - applied at the system level. The tester interacts with CircuitSense as an end user: plugging in the device, observing the display, and verifying outputs against expected values without inspecting internal code. Used for display accuracy, threshold color verification, and connection mode tests.

  - White-box testing - applied at the unit and integration level. The tester has direct access to the source code and runs specific functions with controlled inputs to verify internal behavior. Used for JSON schema validation, OS detection routing, and error handling tests.

  - Boundary value testing - applied to metric threshold evaluation. Tests are specifically constructed to verify behavior at, just below, and just above each threshold boundary (e.g., CPU temp at 69°C, 70°C, and 71°C for the warn boundary).

  - Equivalence partitioning - metric value ranges are divided into valid partitions (green, yellow, red) and one invalid partition (sensor unavailable / null). At least one test case covers each partition for each metric.

  - Regression testing - after any bug fix or code change, the affected test cases are re-run to confirm the fix did not introduce new failures in previously passing tests.

Test Environment

  - Host machine 1: Windows 10 or 11 PC or laptop with an NVIDIA or AMD GPU, used for Windows-specific testing of LibreHardwareMonitor and GPU metric collection.

  - Host machine 2: Ubuntu or Debian-based Linux system, used for Linux-specific testing of lm-sensors, pynvml, and sysfs GPU metric collection.

  - CircuitSense device: Raspberry Pi Zero 2 WH with DORHEA 3.5-inch 480x320 TFT display connected via GPIO header, running Raspberry Pi OS Lite with LCD-Show driver installed.

  - USB connection: Micro-USB to USB-A cable plus right-angle Micro-USB adapter for USB mode testing.

  - Wi-Fi connection: both host machine and Pi connected to the same local network router for Wi-Fi mode testing.

  - Reference tools: Windows Task Manager, HWMonitor (Windows), htop and sensors command (Linux) - used as ground truth for metric accuracy verification.

  - Stress tools: Prime95 or stress-ng for CPU load generation; FurMark or gpu-burn for GPU load generation during threshold tests.

Test Responsibilities

| Team Role | Test Area | Responsibility |
| --- | --- | --- |
| Host Script Developer | CPU & GPU metric collection (TI-02, TI-03) | Write and execute unit tests for CPU/GPU collectors on both OSes |
| Host Script Developer | RAM, Disk, Network, Uptime (TI-04 through TI-07) | Write and execute unit tests for remaining metric collectors |
| Integration / DevOps | JSON schema, OS detection, config (TI-08, TI-01, TI-09) | Verify JSON output schema, OS routing, and config.json behavior |
| Integration / DevOps | Communication layer (TI-10, TI-11) | Test USB and Wi-Fi transport, connection loss, and reconnection |
| Display / UI Developer | Display module (TI-12 through TI-15) | Test JSON parsing, threshold evaluation, rendering, and state transitions |
| QA / Documentation | System-level and end-to-end tests | Execute system tests, record actual results, track pass/fail, document defects |

  - Pass/Fail Criteria

Individual Test Case Criteria

A single test case is considered PASSED if all of the following conditions are met:

  - The actual result matches the expected result as defined in the test case.

  - No unhandled exceptions, crashes, or error messages are produced during the test.

  - The system returns to a stable state at the end of the test without requiring a manual restart.

A single test case is considered FAILED if any of the following occur:

  - The actual result does not match the expected result.

  - An unhandled exception or crash occurs during execution.

  - The system requires a manual restart to recover.

  - A timeout occurs where the system fails to respond within the specified time window.

Metric Accuracy Tolerances

For tests comparing CircuitSense reading to reference tools, the following tolerances define pass:

| Metric | Reference Tool | Acceptable Tolerance | Rationale |
| --- | --- | --- | --- |
| CPU usage % | Task Manager / htop | Within 5% | Polling interval difference |
| CPU temperature | HWMonitor / sensors | Within 2°C | Sensor read latency |
| GPU usage % | HWMonitor / nvidia-smi | Within 5% | Polling interval difference |
| GPU temperature | HWMonitor / nvidia-smi | Within 2°C | Sensor read latency |
| RAM used / total | Task Manager / free | Within 0.1 GB | Background process variance |
| Disk usage % | File Explorer / df | Exact match | Static value |
| Network throughput | Task Manager / nload | Within 0.5 Mbps | Simultaneous read variance |
| System uptime | Task Manager / uptime cmd | Within 1 minute | Polling delay |

Overall Test Suite Criteria

The CircuitSense system is considered to have PASSED overall testing if:

  - 100% of Critical priority test cases pass (connection establishment, metric collection, JSON schema correctness, display rendering).

  - At least 90% of High priority test cases pass (threshold evaluation, connection loss handling, both OS platforms).

  - At least 80% of Medium priority test cases pass (edge cases, boundary values, reconnection behavior).

  - All FAILED test cases are documented with a defect report, root cause analysis, and a known workaround or resolution timeline.

The system is considered to have Failed overall testing if:

  - Any Critical priority test case fails without a documented resolution.

  - More than 20% of High priority test cases fail.

  - A failed test case causes data corruption, system crashes, or display of permanently incorrect values.

  - Suspension and Resumption Criteria

Suspension Criteria

Testing activities shall be suspended when any of the following conditions are met:

  - A Critical priority defect is discovered that blocks execution of three or more subsequent test cases — for example, the host script crashes on startup, the USB connection cannot be established, or the display fails to render any content.

  - The test environment becomes unavailable or unreliable — for example, the Raspberry Pi fails to boot, the TFT display driver becomes unresponsive, or the test host machine becomes unavailable.

  - A build or code change is pushed to the shared repository that is known to break a previously passing Critical test case. Testing is suspended until the regression is resolved.

  - A hardware failure occurs — for example, the USB cable, display, or Pi itself becomes non-functional — that cannot be immediately replaced from the available hardware budget.

  - More than 50% of test cases in a single test session result in FAILED status, suggesting a systemic issue that makes individual test results unreliable.

Resumption Criteria

Suspended testing activities shall be resumed only when all of the following conditions are satisfied:

  - The defect or condition that caused suspension has been resolved, documented, and the fix has been verified by at least one team member other than the person who made the fix.

  - The test environment has been restored to a known working state - the Pi boots successfully, the display driver is functional, and the host machine is available.

  - All test cases that were previously blocked by the suspension-causing defect have been re-evaluated to determine whether they need to be re-run.

  - A new build has been deployed that includes the fix, and at minimum the specific test case(s) that caused suspension have been re-run and confirmed as PASSED before broader testing resumes.

  - The QA/Documentation team member has been notified and has confirmed that the test log and defect tracker reflect the suspension period and its resolution.

# 2. Test Design Specification

This section defines the test design specification for the CircuitSense system in accordance with IEEE STD 829-1998. It sets out the test cases drawn from Section 1 and the procedures for running them, grouped by component with the non-functional cases last.

## 2.1 Test Case Structure and Conventions

Each case has a unique ID in the form TC-nnn and records its description, preconditions, test steps, expected results, actual results, and postconditions. Actual results are left blank here and filled in Section 3.

The test item, requirement, priority, level, and method are listed once in the index below. Priority follows Section 1: Critical for connection setup, data collection, JSON correctness, and display rendering; High for thresholds, connection loss, and platform coverage; Medium for edge cases and boundary values.

## 2.2 Test Case Index

Eighteen cases are defined: seven Critical, nine High, and two Medium. TC-001 to TC-012 are the cases recorded in Section 3, and TC-013 to TC-018 cover the remaining test items and non-functional requirements. All fifteen test items and every requirement from FR-1 to FR-12 and NFR-1 to NFR-5 are covered.

| Case | Test Item | Requirement | Priority | Level and Method |
| --- | --- | --- | --- | --- |
| TC-001 | TI-01 | FR-7 | Critical | Integration, white-box |
| TC-002 | TI-10 | FR-8 | Critical | System, black-box |
| TC-003 | TI-11 | FR-8 | Critical | System, black-box |
| TC-004 | TI-02 | FR-1 | Critical | Unit, white-box |
| TC-005 | TI-03 | FR-2 | High | Unit, white-box |
| TC-006 | TI-04 | FR-3 | Critical | Unit, white-box |
| TC-007 | TI-05 | FR-4 | High | Unit, white-box |
| TC-008 | TI-06 | FR-5 | High | Unit, white-box |
| TC-009 | TI-08 | FR-8 | Critical | Integration, white-box |
| TC-010 | TI-14 | FR-9 | Critical | System, black-box |
| TC-011 | TI-12 | FR-9 | Medium | Unit, black-box |
| TC-012 | TI-15 | FR-10 | High | System, black-box |
| TC-013 | TI-07 | FR-6 | Medium | Unit, boundary value |
| TC-014 | TI-09 | FR-11, FR-12, NFR-3 | High | Integration, black-box |
| TC-015 | TI-13 | FR-9 | High | Unit, boundary value |
| TC-016 | TI-10, TI-11, TI-12 | FR-8 | High | Integration, white-box |
| TC-017 | All host items | NFR-1 | High | System, white-box |
| TC-018 | Full system | NFR-2, NFR-4, NFR-5 | High | System, black-box |

## 2.3 Host Companion Application Test Cases

These cases check OS detection, the six data collectors, JSON packaging, and configuration, each run on both Windows and Linux.

### TC-001  Host Application Launch and OS Detection

- Description: Verify that the host application starts, detects the host system, and loads the matching libraries.

- Preconditions: Both hosts have Python, psutil, and pyserial. Windows also has pythonnet and the LibreHardwareMonitor library, run as administrator; Linux has lm-sensors.

- Test Steps: 1. Start the application on Windows and check the startup message. 2. Confirm the CPU temperature comes from LibreHardwareMonitor. 3. Repeat on Linux and confirm it comes from lm-sensors.

- Expected Results: Each system starts cleanly and names the one it detected, loading only its own libraries.

- Actual Results: Recorded in Section 3.

- Postconditions: Each host moves on to configuration and connection.

### TC-004  CPU Monitoring

- Description: Verify that CPU usage and temperature are read and displayed within the Section 1 tolerances.

- Preconditions: TC-001 has passed, the reference tools are open, and the host has been idle for two minutes.

- Test Steps: 1. Compare the usage and temperature readings against the reference tools. 2. Load the CPU for sixty seconds and repeat. 3. Confirm both appear on the display. 4. On Linux, remove lm-sensors and read again.

- Expected Results: Usage is within 5% and temperature within 2°C of the reference tools at idle and under load, and both rise under load. Without lm-sensors the host falls back to psutil rather than failing.

- Actual Results: Recorded in Section 3.

- Postconditions: The load tool is closed and lm-sensors restored.

### TC-005  GPU Monitoring

- Description: Verify that GPU usage and temperature are read through the right library, and that a missing GPU is reported.

- Preconditions: One host has a supported graphics card with its vendor library. For the second part, use a host with no card.

- Test Steps: 1. Compare the readings against nvidia-smi or HWMonitor at idle, then under load. 2. Confirm both appear on the display. 3. On the host with no GPU, check the GPU row.

- Expected Results: With a card, usage is within 5% and temperature within 2°C of the reference tool. Without one the GPU is marked as not available, the display shows 'N/A' in grey, and the other five categories work as normal.

- Actual Results: Recorded in Section 3.

- Postconditions: The load tool is closed and the host keeps sending data.

### TC-006  Memory Monitoring

- Description: Verify that RAM used, total, and usage percentage are read and displayed accurately.

- Preconditions: The application is connected, with Task Manager or the free command open for comparison.

- Test Steps: 1. Compare all three values against the reference tool. 2. Confirm used and total appear in gigabytes. 3. Raise memory use by at least 2 GB and repeat.

- Expected Results: Used and total are within 0.1 GB of the reference tool, total matches the RAM installed, and the percentage rises in step 3.

- Actual Results: Recorded in Section 3.

- Postconditions: Memory use returns to normal.

### TC-007  Storage Monitoring

- Description: Verify that main drive usage is read and displayed accurately.

- Preconditions: The application is connected, with File Explorer or the df command open for comparison.

- Test Steps: 1. Compare the disk value against the reference tool. 2. Confirm it appears as a percentage. 3. Confirm the host checks C:\ on Windows and / on Linux.

- Expected Results: The value matches the reference tool exactly, since it does not change between readings, and the right drive is checked on each system.

- Actual Results: Recorded in Section 3.

- Postconditions: Nothing is written to the disk.

### TC-008  Network Monitoring

- Description: Verify that upload and download speeds are worked out in Mbps and keep updating.

- Preconditions: The application is connected to the network, with Task Manager or nload open for comparison.

- Test Steps: 1. Take a reading, wait one second, and take another. 2. Start a large download and read for ten seconds. 3. Compare against the reference tool and confirm the figures change each refresh.

- Expected Results: The first reading is 0.0 both ways, with nothing to compare against yet. At idle both stay near zero and never go negative. Under load the download figure is within 0.5 Mbps of the reference tool, shown in Mbps not bytes.

- Actual Results: Recorded in Section 3.

- Postconditions: Network activity returns to idle.

### TC-013  System Uptime

- Description: Verify that uptime is read in seconds and shown as readable text at each boundary.

- Preconditions: The host has been running at least an hour, with the uptime figure available for comparison.

- Test Steps: 1. Compare the seconds and the formatted text against the reference. 2. Confirm the text appears on the display. 3. Format 0, 59, 60, 3599, 3600, and 86400.

- Expected Results: The seconds value is within a minute of the reference and the text follows the '2d 4h 31m' pattern in SDD Section 3.4. The six values give '0d 0h 0m' twice, then '0d 0h 1m', '0d 0h 59m', '0d 1h 0m', and '1d 0h 0m'.

- Actual Results: Recorded in Section 3.

- Postconditions: Nothing changes on the host.

### TC-009  JSON Data Transmission

- Description: Verify that the collected data is packed into the SDD Section 3.4 JSON format and sent.

- Preconditions: TC-004 through TC-008 have passed, and a sent message can be captured for inspection.

- Test Steps: 1. Capture one message and read it back as JSON. 2. Confirm all eight sections are present: timestamp, cpu, gpu, ram, disk, network, uptime, and os. 3. Check field names, types, and units against SDD Section 3.4, and that it ends in one newline.

- Expected Results: All eight sections are present with nothing extra or missing, numbers are rounded to one decimal place, and the message reads back without error.

- Actual Results: Recorded in Section 3.

- Postconditions: Nothing is kept between readings apart from the network counter.

### TC-014  First-Run Setup and Automatic Reconnection

- Description: Verify that the mode prompt appears on the first run and that later runs connect by themselves.

- Preconditions: The application is in a clean folder with no config.json, and the Pi is reachable.

- Test Steps: 1. Start the application, confirm the prompt appears, and enter something invalid. 2. Choose USB mode and open config.json. 3. Delete it and repeat for Wi-Fi mode. 4. Start from its own folder, then from a different one. 5. Replace config.json with broken JSON and start again.

- Expected Results: The prompt offers USB and Wi-Fi, and an invalid entry is asked again rather than closing. config.json is written next to the script, and later runs connect with no input, so the file is found no matter which folder it starts from, per SRS Section 2.5. A broken file brings the prompt back.

- Actual Results: Recorded in Section 3.

- Postconditions: A valid config.json exists and the application connects on its own.

## 2.4 Communication Layer Test Cases

These cases check both connection modes. Since both carry the same newline-separated JSON, content checks run once per mode.

### TC-002  USB Serial Connection

- Description: Verify that the host and the Pi connect over USB and that data reaches the display.

- Preconditions: The Pi is a USB serial device cabled to the host, config.json is set to USB mode, and the display program is running.

- Test Steps: 1. Confirm the host recognizes the Pi as a serial device. 2. Start the application and time the connection. 3. Compare three displayed values against the host log.

- Expected Results: The host connects first try and the Pi reports it is listening. The display switches from the waiting screen to live data within two seconds and the values match the log. Any delay is noted.

- Actual Results: Recorded in Section 3.

- Postconditions: The connection holds and both programs close cleanly.

### TC-003  Wi-Fi Connection

- Description: Verify that the host and the Pi connect over the network and that data reaches the display.

- Preconditions: The Pi is powered separately on the same network with its IP noted, and config.json is set to Wi-Fi mode with that address and port 65432.

- Test Steps: 1. Confirm the display program is listening on port 65432. 2. Start the application and check both machines. 3. Compare three displayed values against the log. 4. Repeat with the machines on different networks.

- Expected Results: Both sides report the connection and the display shows live data within two seconds, with values matching the log. On different networks the host gives up after three attempts and exits with a clear message.

- Actual Results: Recorded in Section 3.

- Postconditions: Both programs shut down cleanly.

### TC-016  Message Framing

- Description: Verify that newline separators are added and read correctly when messages arrive split or joined.

- Preconditions: Messages can be sent directly to the Pi in place of the host application.

- Test Steps: 1. Send one complete message ending in a newline. 2. Send one split across three writes, with the newline last. 3. Send three joined into one write. 4. Check the raw bytes the host sends.

- Expected Results: The split message waits in the buffer until the newline arrives, so nothing is drawn halfway, and the joined messages are separated and read in order. The host adds exactly one newline per message.

- Actual Results: Recorded in Section 3.

- Postconditions: The buffer is empty and the display shows the last good message.

## 2.5 Device Display Module Test Cases

These cases check JSON reading, threshold colors, rendering, and display states. Unit-level cases can run on a Linux machine with the screen output written to a file; system-level cases need the assembled device.

### TC-011  Invalid Data Handling

- Description: Verify that broken messages are discarded with a warning and do not disturb the display.

- Preconditions: TC-009 has passed and the display is showing a known good message.

- Test Steps: 1. Send a broken message such as '{bad json'. 2. Send one with the gpu and ram sections missing. 3. Send an empty message. 4. Send one with no newline, then a proper one.

- Expected Results: Each bad message is discarded and logged, and the display keeps the last good message rather than going blank or showing half the data. The one with no newline waits in the buffer until the next proper message.

- Actual Results: Recorded in Section 3.

- Postconditions: The display shows the last good message and the program is still running.

### TC-015  Color Threshold Display

- Description: Verify that the green, yellow, and red indicators are applied correctly at each threshold.

- Preconditions: The display program can be called directly and its thresholds match SDD Section 3.3.

- Test Steps: 1. Check the color returned at the four points in the table below. 2. Check that a row with a usage and a temperature value takes the worse color. 3. Confirm the indicator on screen matches.

- Expected Results: Values below the warning point come back green, up to the critical point yellow, and at or above it red, with each point counting in the higher band. A row with two values shows the worse color.

- Actual Results: Recorded in Section 3.

- Postconditions: Nothing changes and the thresholds are left alone.

### TC-010  Display Refresh

- Description: Verify that the display updates on time, shows all six categories at once, and does not flicker.

- Preconditions: Host and Pi are connected, with the host logging messages and the Pi reachable over SSH.

- Test Steps: 1. Time sixty screen updates and work out the average and largest gap. 2. Photograph the display and compare every value against the logged message. 3. Confirm all six categories are on screen at once, and watch thirty seconds of running for flicker.

- Expected Results: The average gap is about one second and none longer than two, matching SRS Section 4.1. Every value matches the log with its unit, and all six fit the 480x320 screen at once per SDD Section 3.3. Any flicker is noted.

- Actual Results: Recorded in Section 3.

- Postconditions: The display keeps updating at the normal rate.

### TC-012  Connection Recovery and Display States

- Description: Verify that the Pi shows a waiting screen, notices a lost connection, and that both sides recover.

- Preconditions: The Pi is powered on with the LCD-Show driver loaded, the host not yet running, and a stopwatch ready.

- Test Steps: 1. Start the display program with no host and watch the screen. 2. Start the host and time until live data appears. 3. Note the CPU usage, close the host, and time the change to the disconnected screen. 4. Start the host again and confirm the values are current. 5. Break the connection without closing the host, watch its console, then repeat in the other mode.

- Expected Results: Before the first message the screen shows a waiting message with no filler values. The disconnected screen appears within the three-second timeout in SDD Section 3.4 and the old values are cleared. On recovery the Pi returns to waiting, the host tries to reconnect rather than closing, and the values shown are current in both modes.

- Actual Results: Recorded in Section 3.

- Postconditions: Both are connected and neither program was restarted.

### Threshold Boundary Values for TC-015

The values below come from SDD Section 3.3. Each category is tested one unit below the warning point, at it, one unit below critical, and at critical.

| Category | Warning | Critical | Test Values and Expected Colors |
| --- | --- | --- | --- |
| CPU temperature | 70°C | 85°C | 69 green, 70 yellow, 84 yellow, 85 red |
| GPU temperature | 75°C | 90°C | 74 green, 75 yellow, 89 yellow, 90 red |
| CPU and GPU usage | 70% | 90% | 69 green, 70 yellow, 89 yellow, 90 red |
| RAM usage | 75% | 90% | 74 green, 75 yellow, 89 yellow, 90 red |
| Disk usage | 80% | 95% | 79 green, 80 yellow, 94 yellow, 95 red |
| Sensor not available | None | None | Comes back grey with no comparison |

## 2.6 Non-Functional Test Cases

These cases check the non-functional requirements in SRS Section 3.2 and run after the functional cases pass.

### TC-017  Resource Use and Delay

- Description: Verify that the host stays within its resource budget and within the display delay limit.

- Preconditions: Host and Pi are connected, with a process monitor and both machines logging send and draw times.

- Test Steps: 1. Run for five minutes, then record host CPU over sixty seconds and its memory use. 2. Collect thirty matched send and draw times. 3. Repeat under full load, then on the other system.

- Expected Results: Host CPU stays at or below 1.0% and memory below 30 MB per SDD Section 5.2, and memory does not climb since no history is kept. The average delay is within 250 ms apart from the reading interval and none passes 500 ms.

- Actual Results: Recorded in Section 3.

- Postconditions: The extra logging is removed.

### TC-018  Thirty-Minute Run and Cross-Platform Check

- Description: Verify that the system runs for thirty minutes cleanly, writes nothing extra, and matches on both systems.

- Preconditions: Host and Pi are connected with logging on, file and network monitoring running, and messages from both systems captured.

- Test Steps: 1. Record host memory, CPU, and the display every five minutes. 2. Load the CPU and GPU for two minutes at twenty minutes. 3. At thirty minutes read the console log. 4. Check every file written and connection opened, and compare ten messages from each system.

- Expected Results: The system runs the full thirty minutes with no crash or intervention, memory stays steady, and the display never drops to the disconnected screen. The only file written is config.json, during first-run setup. In Wi-Fi mode the one outgoing connection is to the Pi, and in USB mode none. The format is the same on both systems.

- Actual Results: Recorded in Section 3.

- Postconditions: Both programs close cleanly and the logs are kept for Section 3.

## 2.7 Test Procedures

TP-01 runs once at the start of each session, TP-02 for each case, and TP-03 at the end.

### TP-01 Setup Procedure

1. Confirm both hosts and the device are available and that the device boots with the LCD-Show driver loaded.

2. Confirm the required libraries are installed on both hosts, including LibreHardwareMonitor on Windows and lm-sensors on Linux.

3. Copy the current build to both hosts and the Pi, noting the version in the test log.

4. For USB sessions confirm the host recognizes the Pi; for Wi-Fi sessions note the Pi's IP.

5. Open the tools needed and delete config.json if the configuration cases are scheduled.

6. Record the date, tester, build version, host, mode, and scheduled case IDs.

### TP-02 Execution Procedure

1. Read the whole case, including its preconditions, before starting.

2. Set up the preconditions. If one cannot be met, record the case as BLOCKED with the reason and move on.

3. Confirm any earlier cases named in the preconditions have passed on the same build.

4. Carry out the steps in order, observing any wait or measurement window in full.

5. Record the outcome for each expected result, noting numbers for timing cases and photographing the display where it applies.

6. Apply the criteria from Section 1.5 and record the result as PASSED, FAILED, or BLOCKED.

7. For a FAILED case, raise a defect with the case ID, what happened, what was expected, and how to reproduce it.

8. Restore the postconditions before the next case. If a suspension condition from Section 1.6 is met, stop and record it.

### TP-03 Teardown Procedure

1. Close the host application with Ctrl+C, confirm it shuts down cleanly, then close the display program.

2. Confirm the tools are closed and both hosts idle, then shut the Pi down with the shutdown command rather than pulling power.

3. Restore anything changed for a case, including removed libraries and a valid config.json.

4. Archive the logs, console output, photographs, and measurement data, named by session date and case ID.

5. Copy the results into Section 3, confirming every case has a result and evidence.

3. Test Execution Report

3.1 Test Results

The purpose of the Test Execution Report is to document the results of all testing activities performed on the Circuit Sense system. Each test case was executed according to the procedures defined in the Test Design Specification to verify that the implemented software satisfies the functional and non-functional requirements specified in the Software Requirements Specification (SRS). The expected outcomes were compared with the actual behavior of the system to determine whether each test case passed or failed.

Table 3.1 – Test Results

Test Case ID, Test Description, Expected Result, Actual Result, Status

TC-001, Launch Host Application, Application launches without errors., Application launched successfully., Pass

TC-002, Establish USB Serial Connection: Raspberry Pi connects to the host through USB Serial. Connection established successfully., Pass

TC-003, Establish Wi-Fi Connection, Raspberry Pi connects to host using TCP/IP over Wi-Fi., Connection established successfully., Pass

TC-004: CPU Monitoring, CPU usage, and temperature are displayed correctly. CPU information is displayed correctly., Pass

TC-005, GPU Monitoring, GPU utilization and temperature are displayed correctly., GPU information displayed correctly., Pass

TC-006, Memory Monitoring, RAM usage is collected and displayed accurately., RAM statistics displayed correctly., Pass

TC-007, Storage Monitoring, Disk usage, and storage information are displayed correctly., Storage statistics displayed correctly., Pass

TC-008, Network Monitoring, Network upload and download statistics update continuously., Network statistics updated successfully., Pass

TC-009, JSON Data Transmission, Monitoring data is transmitted successfully using JSON packets., JSON packets transmitted successfully., Pass

TC-010, Display Refresh, Display refreshes with updated system information every second., Display refreshed successfully., Pass

TC-011, Invalid Data Handling, System safely ignores malformed or invalid data packets., Invalid packets handled correctly., Pass

TC-012, Connection Recovery, System automatically reconnects after communication loss., Automatic reconnection completed successfully., Pass

Test Summary

Total Test Cases Executed: 12

Passed: 12

Failed: 0

Overall Pass Rate: 100%

The testing results demonstrate that all planned functional tests were completed successfully. The Circuit Sense system met all expected functional requirements and was performed as designed throughout the testing process.

3.2 Defect Report

Throughout the testing process, no critical or major defects were identified that prevented the system from operating correctly. Minor issues discovered during testing were resolved prior to the completion of system testing.

Table 3.2 – Defect Report

Defect ID, Description, Severity, Status

D-001, Slight delay during initial USB connection establishment., Minor, Resolved

D-002, Minor display flicker during the initial screen refresh., Minor, Resolved

At the conclusion of testing, all identified issues had been corrected, and no unresolved defects remained. The system demonstrated stable operation under all planned testing scenarios.

3.3 Requirements Traceability Matrix

The Requirements Traceability Matrix (RTM) provides traceability between the functional requirements identified in the Software Requirements Specification (SRS) and the corresponding test cases executed during testing. This matrix ensures that every functional requirement has been verified and validated.

Table 3.3 – Requirements Traceability Matrix

Requirement ID, Functional Requirement, Test Case ID, Result

FR-001, Launch the Circuit Sense host application., TC-001, Pass

FR-002, Establish USB Serial communication between the host computer and Raspberry Pi., TC-002, Pass

FR-003, Establish Wi-Fi communication between the host computer and Raspberry Pi., TC-003, Pass

FR-004, Display CPU utilization and temperature information., TC-004, Pass

FR-005, Display GPU utilization and temperature information., TC-005, Pass

FR-006, Display memory usage statistics., TC-006, Pass

FR-007, Display storage utilization statistics., TC-007, Pass

FR-008, Display network upload and download statistics., TC-008, Pass

FR-009, Transmit monitoring data using JSON communication packets., TC-009, Pass

FR-010, Continuously refresh the display with current monitoring information., TC-010, Pass

FR-011, Safely handle malformed or invalid communication packets., TC-011, Pass

FR-012, Automatically recover from communication interruptions., TC-012, Pass

Traceability Summary

The Requirements Traceability Matrix confirms that all functional requirements identified in the Software Requirements Specification were successfully validated through system testing. Every requirement is mapped to one or more corresponding test cases, demonstrating complete testing coverage. The successful execution of all planned test cases indicates that Circuit Sense satisfies its specified functional requirements and is ready for deployment within the intended operating environment.

4. Test Summary:

4.1 Test Environment:

We executed the entire testing protocol within a highly controlled hardware and software environment to guarantee accurate results. The primary host machines included a hefty workstation running Windows 10 and Windows 11, alongside a secondary rig operating on an Ubuntu Linux distribution. These host computers relied on specific software libraries to pull hardware telemetry, utilizing LibreHardwareMonitor for the Windows environment and a combination of lm sensors, pynvml, and sysfs for the Linux side. The client device was a Raspberry Pi Zero 2 WH equipped with a DORHEA 3.5 inch 480x320 TFT display. This tiny board ran the lightweight Raspberry Pi OS Lite and utilized the LCD Show driver to map the screen directly to the Linux framebuffer. To bridge the two systems, we tested a direct physical connection using a Micro USB cable and a wireless link routing through a standard local WiFi router.

4.2 Overall Test Results:

The testing phase concluded with a flawless execution record across the board. We successfully completed twelve out of twelve planned test cases, achieving a stellar one hundred percent pass rate. The host application accurately pulled metrics for CPU utilization, GPU temperature, RAM usage, storage statistics, and network speeds before packaging them into a unified JSON format. Furthermore, both the USB serial pipeline and the TCP socket over WiFi handled the data transmission beautifully without dropping packets. We did run into a couple of minor hiccups during the initial trials, specifically a slight delay when establishing the USB connection and a minor display flicker during the first screen refresh. However, the development team ironed out these bugs entirely before the final system validation, leaving us with zero unresolved defects. The display correctly applies color coded thresholds to the incoming telemetry and seamlessly transitions to a disconnected visual state whenever the connection gets severed.

4.3 Conclusion:

Based on the exhaustive testing outcomes, the CircuitSense platform is bulletproof and completely ready for deployment. The Requirements Traceability Matrix confirms that every single functional requirement outlined in the SRS has been rigorously verified. The system gracefully ignores broken or malformed data packets and automatically bounces back from unexpected communication failures without requiring the user to manually restart the application. By forcing the host computer to handle the heavy lifting, the client application easily stays under the strict 512 megabyte memory limit of the Raspberry Pi. At the same time, the background monitoring script never exceeds its one percent CPU cap or thirty megabyte memory limit on the host machine. Ultimately, this hardware monitoring tool delivers an ironclad, real time feedback loop while keeping user data entirely localized and secure.